WHEELED BIPED ROBOT

Complete Project Reference — v3.0

STM32F411 + ESP32-S3 | Cascade PID → LQR | FSM Gait | FOC Motor Drive | ELRS Long Range

22 Sections — Full Design Session Reference

# 1. Robot Overview

Type: Wheeled Biped — a two-legged robot that can both balance on wheels (segway-style) and step-walk using servo-driven hip and knee joints.

## 1.1 Mechanical Configuration

| Component | Quantity | Description |
| --- | --- | --- |
| Legs | 2 | Symmetric arms either side of main chassis |
| Hip servo | 2 | Forward/backward rotation only — 1 DOF per hip |
| Knee servo | 2 | Flexion/extension — 1 DOF per knee |
| Wheel-foot motor | 2 | TT motor or brushless gimbal motor at foot |
| Main cage | 1 | Central chassis from STEP file |
| Base plate | 1 | Lower chassis structure |
| Servo brackets | 2 | Mirrored motor/servo mounts |

## 1.2 Operating Modes

| Mode | Description | Balance Active | Legs Used |
| --- | --- | --- | --- |
| IDLE | Standing still, holding position | Yes | Parked straight |
| DRIVE | Segway-style wheeled drive at normal speed | Yes | Parked or tucked |
| RACE | High-speed wheeled drive, legs in optimised pose | Yes — high-gain tuning | Tucked for low CoM |
| WALK | Stepping gait, weight-shift locomotion | Yes (modified) | Actively stepping |
| SIT | Emergency lower to ground — safe state | No | Folding down |

## 1.3 Key Design Decisions

- Drive, race and walk are mutually exclusive modes. FSM transitions between them through a neutral STAND state.

- Balance loop runs at all times during drive and race. The 1kHz ISR is untouchable — gait and comms run around it.

- Dual MCU architecture. STM32 handles all hard real-time control. ESP32-S3 handles Bluetooth (short range) or relays ELRS commands (long range).

- Racing is wheeled only. No gait during race mode. Servos position legs for optimal CoM height and weight distribution at speed, then hold position.

- PID first, then LQR. Cascade PID to get the robot balancing, LQR for quality. Switchable at runtime.

# 2. Hardware Selection

## 2.1 Microcontrollers

### Primary MCU — STM32F411CEU6 (Black Pill)

| Parameter | Value |
| --- | --- |
| Architecture | ARM Cortex-M4 |
| Clock speed | 100MHz |
| FPU | Yes — single precision, 1 cycle float ops |
| RAM | 128KB SRAM |
| Flash | 512KB |
| Timers | 11 |
| SPI | 3 ports |
| I2C | 3 ports |
| UART | 6 ports |
| Cost | £3–4 (WeAct Studio on AliExpress) |

Upgrade Path STM32F405RGT6 — 168MHz, 192KB RAM. Use for 4-limb expansion. STM32H743 — 480MHz, 1MB RAM, double FPU. Use for racing high-gain loops or CPG + 12 servo 3-DOF.

### Secondary MCU — ESP32-S3

| Parameter | Value |
| --- | --- |
| Architecture | Dual-core Xtensa LX7 |
| Clock speed | 240MHz |
| Bluetooth | BT5.0 + BLE — Bluepad32 game controller for short range |
| WiFi | 802.11 b/g/n — telemetry and OTA firmware update |
| Role in racing | Receives ELRS CRSF packets via UART, forwards to STM32 |
| Cost | £3–5 (Espressif DevKitC-1) |

WiFi/BT Radio Interference Never run balance loop on the ESP32 — always on STM32. Disable WiFi entirely during racing — shared radio with BT causes scheduler jitter.

## 2.2 IMU — ICM-42688-P

| Parameter | Value |
| --- | --- |
| Interface | SPI up to 24MHz |
| Gyro noise | 0.0028°/s/√Hz — very low noise floor, critical for racing |
| Output rate | Up to 32kHz |
| Data ready pin | Yes — DRDY interrupt for deterministic reads |
| Why over MPU-6050 | Significantly lower noise, faster ODR, better vibration rejection at racing speeds |
| Cost | £3–6 on breakout board |

## 2.3 Encoders and Motor Drivers

| Component | Part | Use |
| --- | --- | --- |
| Encoder (walk/slow) | AS5600 — I2C, 12-bit | Odometry and velocity for balance PID |
| Encoder (racing/FOC) | AS5047P — SPI, 14-bit | Higher resolution, faster reads for FOC torque control |
| Motor driver (TT) | TB6612FNG or DRV8833 | PWM H-bridge for brushed TT motors |
| FOC driver (gimbal) | SimpleFOC Mini or TMC6300 | Brushless FOC for gimbal motors |
| FOC driver (racing) | DRV8323 on custom PCB | High current, built-in current sense amps |

# 3. Motor Selection

## 3.1 EduBot — TT Gear Motors

| Property | Value | Impact |
| --- | --- | --- |
| Voltage | 3–6V | Needs separate regulated supply |
| Speed | ~200 RPM no-load | Fine for walking, too slow for racing |
| Control | PWM speed only | No torque control |
| Encoder | None — add AS5600 externally | Required for balance |
| Verdict | V1 prototyping only | Replace with brushless for ResearchBot |

## 3.2 ResearchBot Walk Modes — Gimbal Motors

| Motor | KV | Stator | Weight | Peak I | Cost | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| GB2208 | 80KV | 22×8mm | ~50g | 3A | £8–12 | Best budget — slow precise torque |
| GM3506 | 80KV | 35×6mm | ~70g | 5A | £12–16 | More torque — heavier legs |
| GM4108H-120T | 120KV | 41×8mm | ~85g | 8A | £12–18 | Good for hub-style wheel |
| T-Motor GB54 | 90KV | 54×14mm | ~110g | 10A | £15–20 | High quality, well characterised |

## 3.3 ResearchBot Race Mode — Higher KV Motors

For wheeled racing the motor must operate efficiently at higher speeds. Gimbal motors (80KV) run at ~4% of their speed range at racing speeds — FOC becomes unreliable at this extreme low end.

| Motor | KV | At 4S (14.8V) Free RPM | At 3m/s (65mm wheel) RPM | Operating % | Notes |
| --- | --- | --- | --- | --- | --- |
| GM3506 gimbal | 80KV | 1184 RPM | 885 RPM | 75% — marginal for racing | Usable if top speed is 1.5 m/s max |
| T-Motor MN2204 | 1400KV | 20720 RPM | 885 RPM | 4% — unusable | Too high KV, needs large reduction |
| iPower GM4108H | 120KV | 1776 RPM | 885 RPM | 50% — acceptable | Best compromise for walk+race |
| T-Motor MN1004 | 400KV | 5920 RPM | 885 RPM | 15% — needs 3:1 reduction | With gearbox: good racing motor |
| 5008 + 10:1 cycloidal | 330KV→33KV eff. | 4880→488 RPM eff. | 885 RPM | 18% — marginal but viable | Cycloidal makes 5008 usable for racing only |

Motor Selection Rule for Walk + Race A motor that works well for both walking (low speed, high torque) and racing (high speed) does not exist without gearing. Best compromise: iPower GM4108H-120T — 50% operating range at racing speed, adequate torque at walking speed. Best performance: dedicated racing motor (MN1004 400KV) with 3:1 reduction — but requires gearbox. For V1: start with GM3506. Accept 1.5 m/s as racing speed limit. Upgrade motor for V2.

## 3.4 T-Motor 5008 + Cycloidal Reducer Analysis

| Aspect | Without Reducer | With 10:1 Cycloidal Reducer |
| --- | --- | --- |
| Operating RPM at 0.5 m/s | 147 RPM (4% of max) | 147 RPM (40% of effective max) |
| FOC viability at walking speed | Very poor — current sense resolution fails | Improved — motor at usable operating point |
| Top speed with 65mm wheel | 12.5 m/s (theoretical) | 1.25 m/s output |
| Weight per motor | 96g | 96g + 80–150g reducer = 180–250g |
| Size | 50mm diameter | 50mm + 30–50mm axial |
| Manufacturing | None | 3D print or machine cycloidal disk |
| Cost vs gimbal motor | Higher total system cost | Higher total system cost |
| Verdict | Wrong motor for this robot | Viable for racing wheeled mode only — not for walking |

# 4. Software Architecture

## 4.1 System Overview

Game Controller / ELRS Receiver ¦ ESP32-S3 • Short range: Bluepad32 BT (PS4/PS5/Xbox) • Long range: ELRS CRSF packet relay from UART to STM32 ¦ ¦ UART 460800 baud (9-byte command packets) ¦ STM32F411 ┌──────────────────────────────────────────┐ │ TIM6 ISR @ 1kHz (Priority 0 — sacred) │ │ IMU read → attitude filter │ │ Balance PID or LQR → motor output │ │ Encoder update │ ├──────────────────────────────────────────┤ │ TIM7 ISR @ 50Hz (Priority 1) │ │ Gait FSM → servo interpolators │ │ Leg pose manager (race mode) │ │ UART packet processing │ ├──────────────────────────────────────────┤ │ Main loop (background) │ │ Serial PID tuning interface │ │ Watchdog kick │ └──────────────────────────────────────────┘

## 4.2 File Structure

| File | Location | Purpose |
| --- | --- | --- |
| config.h | STM32 Inc/ | All physical constants, pin definitions, initial gains |
| imu.h/cpp | STM32 Src/ | ICM-42688 SPI driver, calibration |
| filter.h/cpp | STM32 Src/ | Complementary and Madgwick filter |
| servo.h/cpp | STM32 Src/ | TIM3 4-channel PWM servo driver |
| motor.h/cpp | STM32 Src/ | TIM1 motor PWM, AS5600 encoder, velocity |
| balance.h/cpp | STM32 Src/ | Cascade PID and LQR balance controller |
| gait.h/cpp | STM32 Src/ | FSM gait sequencer, interpolation, race pose manager |
| comms.h/cpp | STM32 Src/ | UART ring buffer, 9-byte packet receiver |
| tuning.h/cpp | STM32 Src/ | Serial live PID tuning interface |
| stm32_main.cpp | STM32 Src/ | Top-level integration, timer ISR setup |
| esp32_main.cpp | ESP32/ | Bluepad32 + ELRS relay, packet builder, UART send |

# 5. Control Systems

## 5.1 Attitude Estimation Filters

| Filter | Equation | Pros | Cons | When to Use |
| --- | --- | --- | --- | --- |
| Complementary | pitch = α(pitch + gyro×dt) + (1-α)×accel_angle | Simple, fast, easy to understand | Susceptible to motor vibration | Start here — works for walking |
| Madgwick | Quaternion gradient descent | Better dynamic performance, lower noise | More CPU, harder to understand | Switch to once PID is tuned |

## 5.2 Cascade PID

OUTER LOOP (50Hz): Input: velocity error (desired_vel - actual_wheel_vel) Output: target lean angle (radians) Gains: PID_OUTER_KP/KI/KD in config.h INNER LOOP (1kHz): Input: pitch error (actual_pitch - lean_setpoint) Output: motor PWM command Gains: PID_INNER_KP/KI/KD in config.h Yaw: differential — add/subtract turn rate to left/right motors

PID Tuning Order 1. Set KI=0, KD=0. Increase KP until oscillation, back off 30%. 2. Add KD to damp oscillation. D uses gyro reading — not differentiated angle. 3. Add KI to correct steady lean offset. Small values only. 4. Tune outer loop only after inner loop is completely stable. Use serial tuning interface — never reflash for gain changes.

## 5.3 LQR Controller

State vector:

Control law:

import numpy as np from scipy.linalg import solve_continuous_are M=0.8; m=0.6; l=0.12; I=0.002; r=0.0325; g=9.81 den = M*I + m*I + M*m*l*l A = np.array([[0,1,0,0],[m*g*l*(M+m)/den,0,0,0], [-m*m*g*l*l/den,0,0,0],[0,0,1,0]]) B = np.array([[0],[-(I+m*l*l)/den], [(I+m*l*l+m*l*r)/den/r],[0]]) Q = np.diag([100,1,10,1]) # increase Q[0,0] to prioritise tilt R = np.array([[0.01]]) P = solve_continuous_are(A,B,Q,R) K = np.linalg.inv(R) @ B.T @ P print(K) # update LQR_K1..K4 in config.h

## 5.4 Gait FSM

STAND → WEIGHT_SHIFT_LEFT → SWING_RIGHT → PLANT_RIGHT ↓ WEIGHT_SHIFT_LEFT ← SWING_LEFT ← WEIGHT_SHIFT_RIGHT State transitions: minimum duration elapsed AND all joints at target Lean bias: ±0.03 rad during weight-shift states (assists transfer) Interpolation: trapezoidal velocity profile — ramp up, hold, ramp down

# 6. Serial Tuning Interface

Connect via USB-serial to ST-Link VCP at 115200 baud. Commands are ASCII terminated with newline.

| Command | Example | Effect |
| --- | --- | --- |
| ip.kp=VALUE | ip.kp=85.0 | Inner PID P gain |
| ip.ki=VALUE | ip.ki=5.0 | Inner PID I gain |
| ip.kd=VALUE | ip.kd=12.0 | Inner PID D gain |
| op.kp=VALUE | op.kp=2.0 | Outer PID P gain |
| balance=pid | balance=pid | Use cascade PID |
| balance=lqr | balance=lqr | Use LQR controller |
| mode=drive | mode=drive | Switch to drive mode |
| mode=race | mode=race | Switch to race mode (high-gain PID, tucked legs) |
| mode=walk | mode=walk | Switch to walk mode |
| mode=sit | mode=sit | Emergency sit |
| fil.type=0 | fil.type=0 | Complementary filter |
| fil.type=1 | fil.type=1 | Madgwick filter |
| gait.speed=0.6 | gait.speed=0.6 | Walk speed 0.0–1.0 |
| gait.step=30 | gait.step=30 | Knee lift angle (degrees) |
| ? | ? | Print all current values |

# 7. Game Controller — Short Range

## 7.1 Bluepad32 (Bluetooth, up to ~10m)

| Control | Input | Robot Action |
| --- | --- | --- |
| Left stick Y | Forward/back | Drive/race velocity (±0.5 m/s drive, ±3 m/s race) |
| Right stick X | Left/right | Yaw rate (±2.0 rad/s) |
| L1 (hold) | Button | Walk mode while held, drive on release |
| L2 (hold) | Button | Race mode while held — high-gain PID, legs tuck |
| R1 | Button press | Cycle walk speed |
| Cross (X) | Button press | Emergency sit |
| Triangle | Button press | Toggle PID ↔ LQR |
| Square | Button press | Toggle complementary ↔ Madgwick filter |

| Supported Controller | Notes |
| --- | --- |
| PlayStation 4 DualShock | Recommended — best tested with Bluepad32 |
| PlayStation 5 DualSense | Supported |
| Xbox One / Series X | Supported |
| Nintendo Switch Pro | Supported |
| 8BitDo controllers | Supported |

# 8. Custom PCB Guidelines

## 8.1 Power Architecture

LiPo (3S 11.1V walk ¦ 4S 14.8V race) ├─ Reverse polarity protection (P-channel MOSFET) ├─ Bulk capacitor 470–1000µF (absorbs motor spikes) ├─ Motor H-bridges / FOC stage (direct from battery) ├─ 5V buck converter → servos + BEC │ Add 100–470µF on 5V servo rail └─ 3.3V LDO from 5V → STM32, encoders, IMU

Most Common PCB Mistake Powering servos and motors from the same rail. Motor spikes cause voltage droops that reset servos mid-movement. Always use separate rails with separate regulators.

## 8.2 Key STM32F411 Pin Assignments

| Pin | Function | Peripheral | Notes |
| --- | --- | --- | --- |
| PA6 | Servo Hip Left | TIM3_CH1 | 50Hz PWM |
| PA7 | Servo Hip Right | TIM3_CH2 | 50Hz PWM |
| PB0 | Servo Knee Left | TIM3_CH3 | 50Hz PWM |
| PB1 | Servo Knee Right | TIM3_CH4 | 50Hz PWM |
| PA8 | Motor Left PWM | TIM1_CH1 | 168kHz PWM |
| PA9 | Motor Right PWM | TIM1_CH2 | 168kHz PWM |
| PA4 | IMU CS | GPIO out | SPI chip select |
| PA5 | IMU DRDY | GPIO in | Data ready interrupt |
| PB10 | I2C2 SCL | I2C2 | Encoder bus |
| PB11 | I2C2 SDA | I2C2 | Encoder bus |
| PA2 | UART2 TX | USART2 | To ESP32 RX |
| PA3 | UART2 RX | USART2 | From ESP32 TX |

## 8.3 FOC Integration (Custom PCB V2)

| Component | Choice | Notes |
| --- | --- | --- |
| Gate driver | DRV8323 (TI) | Built-in 3-phase, current sense amps, SPI config — best choice |
| MOSFETs | CSD88537 or similar | 40V N-channel, low gate charge, matched pairs |
| Current sense | Low-side 3-shunt 5–10mΩ | Simplest accurate approach — what DRV8323 is designed for |
| Position encoder | AS5047P SPI 14-bit | Required for FOC at low speed — not optional |
| PWM frequency | 20kHz SVPWM | Inaudible, good efficiency, manageable switching loss |

# 9. Recommended Build Order

## 9.1 Hardware Sequence

- Mechanical assembly. Full leg mechanism. Verify range of motion. Add hard stops to all joints.

- Bench test servos. Verify each servo moves correctly with a simple Servo.write() sketch.

- Test TT motors. Verify H-bridge wiring, direction logic.

- Mount IMU. Rigid mount, vibration isolated. Verify SPI communication.

- Add encoders. Verify AS5600 reads valid angles including wrap handling.

- Integrate power. Verify separate servo and motor rails. Check LiPo voltage under load.

## 9.2 Firmware Sequence

- Flash blink sketch. Confirm ST-Link works and clock is running.

- Serial tuning interface. Type "?" and verify response.

- IMU reads. Stream raw accel/gyro. Verify Z-axis reads ~9.81 when level.

- Attitude filter. Stream pitch. Verify it responds when robot is tilted.

- Encoders. Spin wheels by hand. Verify velocity sign and magnitude.

- Motors. Small PWM commands. Verify direction and speed.

- Balance drive mode. Tune inner PID, then outer PID. Use serial interface.

- Game controller. Flash ESP32. Verify UART packets arrive and velocity commands work.

- Race mode. Increase gains, tuck legs, test at increasing speed.

- Gait walk mode. Test each state individually. Verify interpolation is smooth.

- LQR. Measure physical parameters. Run Python script. Flash and compare to PID.

- ELRS. Bind receiver. Verify CRSF packets parse. Test failsafe.

## 9.3 Robot Version Roadmap

| Version | Config | Goal | New Concepts |
| --- | --- | --- | --- |
| V1 | 2 legs, TT motors, 2 DOF | Walking and balancing | PID, FSM, servo control |
| V1.5 | 2 legs, gimbal motors | Torque control, smoother balance | SimpleFOC, LQR |
| V2 | 4 legs, TT or gimbal | CPG gait experiments | CAN bus, gait coupling |
| V2.5 | 4 legs + cycloidal hub | Racing wheeled mode at speed | 5008 + cycloidal, higher KV |
| V3 | 4 legs, 3 DOF/leg | Full 3D locomotion | IK, CPG, STM32H7, MPC |

# 10. Learning Roadmap

## 10.1 Current Position

You are at Stage 3: C++ concepts — currently working through pointers.

## 10.2 Full Learning Path

| Stage | Topic | Duration | Key Outcome |
| --- | --- | --- | --- |
| 1 | Python basics | 2–4 weeks | Variables, loops, functions — programming logic |
| 2 | Arduino C++ basics | 4–6 weeks | LED, servo, motor, sensor — hardware control |
| 3 | C++ concepts (NOW) | 2–4 weeks | Pointers, structs, bit ops, header files |
| 4 | Simple balancer project | 2–3 months | PID working, balance proven before adding legs |
| 5 | STM32duino | 1–2 months | On target hardware, same code style |
| 6 | STM32 HAL + codebase | 3–6 months | Full robot using provided codebase |

## 10.3 Best Resources

| Resource | Topic | Location |
| --- | --- | --- |
| learncpp.com | C++ fundamentals | learncpp.com — best free C++ resource |
| arduino.cc/en/Tutorial | Arduino basics | Official Arduino tutorials |
| Paul McWhorter YouTube | Arduino deep dive | Search "Paul McWhorter Arduino" |
| Fastbit Embedded Brain Academy | STM32 beginner | YouTube search |
| JJRobots B-robot | Simple balancer project | jjrobots.com — full source and guide |
| SimpleFOC docs | FOC implementation | docs.simplefoc.com |
| ExpressLRS docs | ELRS setup | expresslrs.org |

# 11. Safety Systems

## 11.1 Hardware Safety

- Mechanical hard stops on all joints: Prevents servo stripping.

- Separate motor and servo power rails: Motor spikes cannot reset servos.

- Bulk capacitor 470–1000µF on motor rail: Absorbs current transients.

- Low-voltage cutoff on LiPo: 3.0V per cell minimum. 4S pack = 12.0V cutoff.

- XT60 connectors: Lock under vibration. Never barrel jacks.

## 11.2 Firmware Safety

| Feature | Location | What It Does |
| --- | --- | --- |
| MAX_TILT_DEG = 35° | config.h | Cut motors if tilt exceeds this — prevents violent crash recovery |
| CONTROLLER_TIMEOUT_MS | config.h | 200ms without valid packet → zero commands |
| ELRS hardware failsafe | ELRS receiver | Signal loss → preset stop values in <2ms |
| GPS geofence | ESP32/GPS module | Outside boundary → reduce speed limit to 0.5 m/s |
| Return to home | ESP32/GPS | Signal lost >1000ms → navigate back to home GPS point |
| Anti-windup clamp | balance.cpp | Integral clamped to max_integral — prevents windup when picked up |
| MODE_SAFE_SIT | gait.h | Cross button → lowers robot to ground safely |

## 11.3 Racing-Specific Safety

Racing Safety Requirements At 3 m/s a 2kg robot carries ~9 joules of kinetic energy — enough to cause injury. Always race in an open space with a clear safety perimeter. ELRS hardware failsafe is non-negotiable for racing — configure before first test. GPS geofencing prevents runaway if control is lost at range. Tilt cutoff at 35° prevents violent tumbling at speed. Never race near people or animals.

# 12. Quick Reference

## 12.1 config.h Values to Measure

| Constant | Unit | How to Measure | Default |
| --- | --- | --- | --- |
| ROBOT_MASS_KG | kg | Weigh complete assembled robot | 0.8 |
| WHEEL_RADIUS_M | m | Measure wheel diameter / 2 | 0.0325 |
| WHEEL_BASE_M | m | Distance between wheel centres | 0.15 |
| COM_HEIGHT_M | m | Measure CoM height (balance point) | 0.12 |
| COM_HEIGHT_RACE_M | m | CoM height in tucked race pose — measure separately | 0.08 |

## 12.2 PID Starting Points

| Gain | Start Value | Increase If | Decrease If |
| --- | --- | --- | --- |
| ip.kp (inner P) | 50–80 | Too slow to correct tilt | Oscillating |
| ip.kd (inner D) | 8–15 | Still oscillating after P ok | Noisy/jittery |
| ip.ki (inner I) | 1–5 | Steady lean offset remains | Unstable, windup |
| op.kp (outer P) | 1–3 | Slow velocity response | Velocity oscillation |
| RACE ip.kp | 120–200 | Too slow at racing speed | Violent oscillation |
| RACE op.kp | 5–10 | Sluggish velocity tracking | Hunting/overshoot |

## 12.3 Code File Summary

| File | Lines | Key Functions |
| --- | --- | --- |
| config.h | ~120 | All constants — edit this to match your hardware |
| imu.cpp | ~180 | IMU_Init, IMU_ReadRaw, IMU_Calibrate |
| filter.cpp | ~160 | Filter_Init, Filter_Update, Filter_SetType |
| servo.cpp | ~130 | Servo_Init, Servo_SetAngle, Servo_SetAll |
| motor.cpp | ~200 | Motor_Init, Motor_SetBoth, Encoder_Update |
| balance.cpp | ~220 | Balance_Init, Balance_Update, Balance_SetMode |
| gait.cpp | ~280 | Gait_Init, Gait_Update, Gait_SetMode, Race_SetPose |
| comms.cpp | ~100 | Comms_Init, Comms_ProcessIncoming |
| tuning.cpp | ~140 | Tuning_Init, Tuning_Process, Tuning_PrintAll |
| stm32_main | ~170 | TIM6/TIM7 ISR setup, top-level integration |
| esp32_main | ~200 | Bluepad32 + ELRS relay, packet builder, UART send |

# 13. Two Product Versions

EduBot ($50 max): Cheap, educational, teaches everything by hitting limits. Arduino Mega, TT motors, FSM gait on separate firmwares.

ResearchBot (no limit): Personal research platform. STM32F411 + ESP32-S3, gimbal motors, FOC, ELRS, custom PCB, CPG research.

## 13.1 Side-by-Side Overview

| Aspect | EduBot ($50 max) | ResearchBot (no limit) |
| --- | --- | --- |
| Purpose | Teach robotics from scratch at low cost | Personal research platform |
| Target user | Beginners, students, budget constrained | You — after EduBot mastered |
| MCU | Arduino Mega2560 clone (~$5) | STM32F411 + ESP32-S3 |
| Motors | TT gear motors | Brushless gimbal motors + FOC |
| Gait control | FSM on separate firmware | FSM → CPG, unified architecture |
| Balance | Cascade PID, tuned manually | Cascade PID → LQR → MPC research |
| Wireless | HC-05 Bluetooth serial (10m) | ELRS 2.4GHz (5–30km) + Bluepad32 BT |
| Programming | Arduino IDE, single sketch | STM32CubeIDE, HAL/LL, FreeRTOS |
| Build time | 1–2 weekends | 3–6 months |

## 13.2 Version 1 — EduBot ($50 Maximum)

### Bill of Materials

| Component | Part | Qty | Cost | Notes |
| --- | --- | --- | --- | --- |
| MCU | Arduino Mega2560 clone | 1 | $4–6 | Clone not genuine — identical chip |
| IMU | MPU-6050 GY-521 | 1 | $1–2 | Noisier than ICM but sufficient |
| Servos | MG996R or SG5010 | 4 | $8–16 | Metal gear, buy 4-pack |
| Motors | TT gear motor 1:48 | 2 | $2–4 | Standard yellow TT motor |
| Motor driver | L298N module | 1 | $1–2 | Or TB6612 — smaller and better |
| Wheels | 65mm rubber | 2 | $1–2 | Fits TT shaft directly |
| Bluetooth | HC-05 or HC-06 | 1 | $2–4 | Serial UART bridge |
| Chassis | 3D printed PLA | 1 | $3–8 | Print from design files |
| Battery | 2S LiPo 1000mAh | 1 | $5–8 | Keep below 20C rating fine |
| Hardware | Screws, cables, misc | 1 | $4–6 | M3 kit + dupont cables |

Total: $32–58 depending on sourcing. AliExpress pricing used above.

Cost Saving Rules Buy Arduino Mega CLONE not genuine — $4 vs $45, identical chip. Buy MPU-6050 GY-521 MODULE not breakout — $1.50 vs $8. Buy MG996R in 4-pack — cheaper per unit. Print chassis if possible — saves $10–20. Do NOT buy an Uno — Mega is essential. Uno has 2KB RAM, not enough.

### EduBot Curriculum — 22 Weeks

| Week | Project | Concept Learned |
| --- | --- | --- |
| 1–2 | Blink + Serial | setup/loop, serial monitor |
| 3–4 | Servo control | PWM, Servo library, angle to pulse |
| 5–6 | Motor + H-bridge | DC motors, direction, PWM speed |
| 7–8 | MPU-6050 reading | I2C protocol, register reading |
| 9–10 | Complementary filter | Sensor fusion — why raw data fails |
| 11–14 | Walk firmware | FSM design, multi-servo coordination |
| 15–18 | Drive PID balance | PID theory, tuning, cascade loops |
| 19–20 | Bluetooth control | UART comms, packet parsing, timeouts |
| 21–22 | Hitting the limits | Why AVR/Arduino is insufficient — motivation for ResearchBot |
| 23–26 | AVR capstone (optional) | Registers, timer ISR, float profiling — see Section 17 |

## 13.3 Version 2 — ResearchBot

### Smart Servo Upgrade

| Servo | Bus | Torque | Cost | Why |
| --- | --- | --- | --- | --- |
| Feetech STS3215 | SCS serial | 15 kg·cm | £8–12 | Best value smart servo — real-time torque/temp feedback |
| Feetech SCS0009 | SCS serial | 9 kg·cm | £5–8 | Smaller, lighter for knee joints |
| DYNAMIXEL XL330 | TTL | 7.7 kg·cm | £25–30 | Industry standard, excellent docs |

Smart Servo Advantages for Research Real-time torque feedback — measure joint load during walking. Temperature monitoring — detect overloaded joints before failure. Velocity control mode — command joint speed, useful for CPG outputs. All servos share one UART bus — 4 servos need only 1 MCU UART port.

# 14. Research Applications — ResearchBot

The ResearchBot is a configurable research instrument relevant to a wide range of active research fields.

## 14.1 Locomotion and Gait Research

### CPG Gait

| Research Question | What ResearchBot Can Test |
| --- | --- |
| How do coupling weights determine emergent gait? | Vary coupling matrix between left/right and front/back oscillators |
| How does sensory feedback modify CPG? | Feed encoder velocity and foot contact back into oscillator equations |
| Can CPG adapt online to terrain? | Change surface and measure step frequency adaptation |
| How does CPG frequency relate to energy efficiency? | Log motor current vs step frequency at different speeds |

## 14.2 Control Systems Research

- PID vs LQR vs MPC comparison: Apply impulse disturbance, measure recovery time and overshoot for each controller.

- Filter comparison: Complementary vs Madgwick vs Kalman — quantify noise, latency, CPU cost on STM32.

- Gain scheduling: Change PID gains based on mode (walk vs drive vs race) — characterise optimal gains per mode.

- Real-time jitter measurement: Use timer capture to measure actual ISR execution time under load.

## 14.3 Machine Learning

- Sim-to-real transfer: Train balance/gait policy in PyBullet or MuJoCo, transfer to physical robot, measure degradation.

- Domain randomisation: Train with randomised physical parameters — measure improvement in sim-to-real gap.

- TFLite on STM32: Quantise a trained policy to int8. Deploy and measure inference latency vs control loop budget.

- Imitation learning: Record expert joystick control as training data. Train behaviour cloning policy.

## 14.4 Biomechanics and Neuroscience

- Matsuoka oscillator validation: Compare emergent gait to biological recordings from cat locomotion studies.

- Cost of transport: Measure joules per metre at different speeds. Find optimal speed that minimises energy.

- Entrainment to perturbation: Apply rhythmic perturbations, measure how CPG frequency shifts — compare to biological data.

## 14.5 Racing and High-Speed Research

- Balance gain scheduling at speed: Characterise how optimal PID/LQR gains change as robot speed increases from 0 to max.

- Leg pose optimisation for speed: Test different leg tuck configurations at racing speed. Measure stability margin for each.

- Speed limit of wheeled biped: Determine the maximum stable speed as a function of CoM height, wheel base, and control bandwidth.

- ELRS latency characterisation: Measure end-to-end latency from stick input to motor command at different packet rates.

## 14.6 Add-On Hardware Roadmap

| Add-On | Interface | Research Enabled | When |
| --- | --- | --- | --- |
| Foot pressure sensors | ADC/I2C | Ground contact detection, CPG feedback | With V2 PCB |
| Magnetometer (LIS3MDL) | I2C | Absolute heading, full 9-DOF fusion | Early — cheap |
| AS5047P encoders | SPI | Higher resolution FOC feedback | With brushless upgrade |
| Depth camera (OAK-D Lite) | USB3 | Terrain mapping, obstacle avoidance | After walking stable |
| Current monitor (INA226) | I2C | Per-motor power, efficiency research | With V2 PCB |
| GPS (u-blox M9N) | UART | Position, return-to-home, geofence | With ELRS racing |
| SD card logger | SPI | Long-form data for ML training | STM32 has spare SPI |
| LiDAR (RPLIDAR A1) | UART | 2D mapping, navigation | After stable platform |

# 15. ODrive vs FOC — The Distinction

Core Clarification FOC (Field Oriented Control) is an ALGORITHM — a mathematical technique for controlling brushless motors. It exists purely as code and equations. It has no physical form. ODrive is a PRODUCT — a motor controller board that runs FOC internally. "ODrive vs FOC" is like saying "iPhone vs touchscreen" — the iPhone uses a touchscreen. Every serious brushless motor controller today runs FOC underneath.

## 15.1 The Full Ecosystem

FOC (the algorithm) ├── SimpleFOC open source library — runs on Arduino/STM32 ├── SimpleFOC Mini small PCB designed to run SimpleFOC (£8–12) ├── ODrive commercial board running FOC (£100–200) ├── VESC open source board running FOC (£60–100) ├── Moteus open source, CAN-native FOC (£80–100) └── Your custom PCB also runs FOC — same algorithm, your hardware

## 15.2 Full Comparison

| Approach | What It Is | Cost/axis | Complexity | Best For |
| --- | --- | --- | --- | --- |
| SimpleFOC library | Open source FOC, Arduino/STM32 | £0 (SW) | Low–mod | Learning FOC, small motors |
| SimpleFOC Mini | Small PCB for SimpleFOC library | £8–12 | Low | EduBot upgrade, gimbal motors |
| ODrive S1 | Commercial single-axis FOC | £100–120 | Low–mod | High performance, plug and play |
| VESC 6 | Open source FOC, most popular | £60–100 | Moderate | Widely supported, flexible |
| Moteus r4.11 | Open source, CAN native | £80–100 | Moderate | Multi-axis CAN systems |
| Custom + DRV8323 | Your board, FOC on STM32 | £8–15 | High | ResearchBot V2 PCB |

## 15.3 Per-Robot Recommendation

| Version | Motor | FOC Approach | Reason |
| --- | --- | --- | --- |
| EduBot | TT motor | None — L298N PWM | Arduino Mega cannot run FOC. PWM speed control sufficient. |
| EduBot upgrade | GM2208 | SimpleFOC Mini | Cheap, educational, readable source code |
| ResearchBot V1 | GM3506 | SimpleFOC Mini | Proven, replaceable with custom PCB later |
| ResearchBot V2 | GM3506/custom | Custom PCB DRV8323 | Full stack ownership |
| ResearchBot V3 | 4+ axis | Moteus or CAN custom | At 4+ motors CAN bus essential |

## 15.4 FOC Algorithm Summary

3-phase currents (Ia, Ib, Ic) ← current sense resistors ↓ Clarke transform 2-phase stationary (Iα, Iβ) ↓ Park transform (needs rotor angle θ from AS5047P encoder) Rotating frame (Id, Iq) ↓ Two PI controllers Vd (→ 0) and Vq (→ torque_cmd in Newton-metres) ↓ Inverse Park → Inverse Clarke Space Vector PWM → 6 MOSFET gate signals Pipeline runs at 20,000–50,000 Hz — much faster than balance loop

# 16. STM32 Programming Levels

Unlike Arduino where there is one level, STM32 has four distinct programming levels. Understanding which to work at is one of the most important decisions for ResearchBot development.

Level 1 — STM32duino (Arduino API on STM32 hardware) Level 2 — STM32 HAL (Hardware Abstraction Layer — what the codebase uses) Level 3 — STM32 LL (Low Layer — thin register wrappers) Level 4 — Bare Metal (direct register writes, no driver layer)

| Level | Abstraction | Performance | Dev Speed | Codebase Compatible | Required |
| --- | --- | --- | --- | --- | --- |
| STM32duino | High | Good | Fast | No — needs rewrite | No — transition only |
| HAL | Medium | Very good | Medium | Yes — primary target | Yes — minimum |
| LL | Low | Excellent | Slower | Yes — alongside HAL | Optional |
| Bare metal | None | Maximum | Slowest | Partial — CCR writes | No — educational |

## 16.1 Level 1 — STM32duino

Necessary? No. Useful? Yes — 2–4 weeks transition from EduBot.

Use STM32duino to Validate every hardware connection — IMU, servos, motors, encoders. Run Bluepad32 controller immediately. Use familiar Arduino libraries for rapid prototyping. Then move to HAL once hardware is verified.

Cannot do at STM32duino level Configure interrupt priorities properly — NVIC control not exposed. Access DMA for IMU reads. The codebase as written will not compile without significant rewriting.

## 16.2 Level 2 — STM32 HAL

Necessary? Yes — this is the minimum for ResearchBot. The codebase targets this level.

// HAL peripheral access — full control, type-safe HAL_SPI_Transmit(&hspi1, &reg_addr, 1, 5); HAL_SPI_Receive(&hspi1, buf, 6, 5); // NVIC priority — HAL exposes this properly HAL_NVIC_SetPriority(TIM6_DAC_IRQn, 0, 0); // priority 0 = highest HAL_NVIC_EnableIRQ(TIM6_DAC_IRQn); // Timer interrupt start HAL_TIM_Base_Start_IT(&htim6); // fires 1kHz balance loop ISR

## 16.3 Level 3 — STM32 LL

Necessary? No — but cleanest way to write performance-critical runtime code.

// Three ways to update servo CCR — all identical performance: // Direct register (what codebase uses) TIM3_CCR1 = 1500; // LL driver (cleaner, same speed, named constants) LL_TIM_OC_SetCompareCH1(TIM3, 1500); // HAL (slightly slower — checks params, updates state) __HAL_TIM_SET_COMPARE(&htim3, TIM_CHANNEL_1, 1500);

## 16.4 Level 4 — Bare Metal

Necessary? No. Educational? Yes — for specific peripherals, one at a time.

// Bare metal timer enable — no HAL, no LL RCC->APB1ENR ¦= RCC_APB1ENR_TIM3EN; // enable clock TIM3->PSC = 83; // 84MHz/84 = 1MHz TIM3->ARR = 19999; // 50Hz TIM3->CCMR1 ¦= (TIM_CCMR1_OC1M_2 ¦ TIM_CCMR1_OC1M_1); TIM3->CCER ¦= TIM_CCER_CC1E; TIM3->CR1 ¦= TIM_CR1_CEN; // This is exactly what HAL_TIM_PWM_Start() does internally

## 16.5 Recommended Path for ResearchBot

| Phase | Level | Duration | Goal |
| --- | --- | --- | --- |
| 1 | STM32duino | 2–4 weeks | Validate all hardware, familiar syntax |
| 2 | HAL + CubeMX | 2–3 months | Full codebase running, balance loop stable |
| 3 | HAL + direct CCR | Ongoing | Performance-critical paths optimised |
| 4 | LL for complex peripherals | As needed | Cleaner register code for SPI/I2C |
| 5 | Bare metal (1 peripheral only) | 1 afternoon | Understand what HAL_TIM_Base_Init does internally |

## 16.6 Skills Required for ResearchBot

| Skill | Required | How |
| --- | --- | --- |
| Reading STM32 reference manual | Yes | Open RM0383 (F411). Cross-reference with codebase timer setup. |
| Timer CCR register writes | Yes | Already in servo.cpp — understand the prescaler/ARR/CCR relationship |
| NVIC priority config | Yes | HAL_NVIC_SetPriority — 0 = highest, understand why balance loop is 0 |
| CubeMX peripheral config | Yes | Follow any CubeMX tutorial — 2 hour investment, saves weeks of config |
| DMA configuration | Optional | HAL DMA for IMU reads — improves efficiency, not required for V1 |
| Full bare metal init | No | CubeMX generates this correctly — educational curiosity only |

# 17. AVR — Is It Necessary

## 17.1 Necessity by Robot Version

| Robot | Necessary? | Reason |
| --- | --- | --- |
| EduBot | No — but valuable as late-stage capstone | Uses Arduino Mega (ATmega2560). Understanding underlying registers is optional but very educational. |
| ResearchBot | No — completely irrelevant | Uses STM32F411 (ARM Cortex-M4). Different architecture, different company. Nothing transfers directly. |

The Key Insight AVR (ATmega) and STM32 (ARM Cortex-M) are completely different architectures. Different register names, peripheral models, and instruction sets. The VALUE of AVR for EduBot is not the registers themselves — it is the concept: "Hardware peripherals are memory-mapped registers, and Arduino is hiding them." That CONCEPT transfers to STM32 even though the specific registers do not.

## 17.2 What Happens If You Skip AVR

| Path | Time Saved | Knowledge Gap | Robot Impact |
| --- | --- | --- | --- |
| Skip AVR → Arduino → STM32duino → HAL | 4–6 weeks | Shallow understanding of abstraction layers. Closes naturally with STM32 experience. | None — robot works identically |
| Include AVR capstone | 0 (costs 4–6 weeks) | None — deep understanding of hardware abstraction | None — but STM32 registers feel familiar |
| Single AVR timer exercise only (recommended) | 3–5 weeks | Minimal — one peripheral is enough for the concept | None |

## 17.3 The Recommended Compromise — One Exercise

// Replace TimerOne library with raw AVR registers // This reveals exactly what TimerOne.initialize() was hiding // BEFORE (black box): Timer1.initialize(2000); Timer1.attachInterrupt(balanceLoop); // AFTER (glass box): TCCR1A = 0; TCCR1B = (1<<WGM12)¦(1<<CS11)¦(1<<CS10); // CTC mode, prescaler 64 OCR1A = 499; // 16MHz/64/500 = 500Hz TIMSK1 = (1<<OCIE1A); sei(); ISR(TIMER1_COMPA_vect) { balanceLoop(); } // After writing this: open TimerOne source at github.com/PaulStoffregen/TimerOne // You will see exactly this pattern. The library was never magic.

## 17.4 Full AVR Capstone (Optional 4 Weeks)

| Week | Topic | Exercise | Concept |
| --- | --- | --- | --- |
| 23 | Port manipulation | Blink pin 13 via PORTB register. Measure speed vs digitalWrite with logic analyser. | Variables at fixed addresses control physical pins. digitalWrite hides ~125 lines of C. |
| 24 | Direct timer ISR | Replace TimerOne balance loop with raw TCCR/OCR/TIMSK. Read TimerOne source after. | Prescaler, compare register, interrupt flag — same pattern conceptually as STM32 timers. |
| 25 | Direct PWM | Replace analogWrite for motors with direct OCR register writes. | PWM is a compare register and a counter. High-level functions add overhead and hide config. |
| 26 | Float performance wall | Profile balance loop with GPIO toggle + logic analyser. Compare float vs integer. | No FPU = float multiply costs 12 cycles. STM32 costs 1 cycle. This is why FPUs exist. |

## 17.5 Float Performance Wall — The Key Measurement

// Profile on ATmega2560 using GPIO toggle ISR(TIMER1_COMPA_vect) { PORTB ¦= (1<<PB4); // GPIO high — measure this with logic analyser // ... balance loop with float math ... PORTB &= ~(1<<PB4); // GPIO low } // Typical results: // ATmega2560 @ 16MHz, float: ~800µs → 500Hz ceiling // ATmega2560 @ 16MHz, fixed-point: ~120µs → 1kHz possible but code is ugly // STM32F411 @ 100MHz, float + FPU: ~0.08ms → 12,500Hz possible // Improvement: 60× headroom from one architectural decision // This measurement, done yourself, is worth more than being told "STM32 is faster"

AVR Knowledge That Does NOT Transfer Register names (TCCR1A, OCR1A, TIMSK1) — completely different on STM32. Interrupt vector names (TIMER1_COMPA_vect) — STM32 uses NVIC. Port manipulation (PORTB, DDRB) — STM32 uses GPIO->ODR, GPIO->MODER. AVR-specific peripherals (USI, TWI) — do not exist on STM32.

AVR Knowledge That DOES Transfer The concept: peripherals are memory-mapped registers. The pattern: prescaler + compare register + interrupt flag = timer ISR. The habit: reading datasheets and reference manuals. The instinct: questioning what high-level functions are hiding. The measurement mindset: profile with GPIO toggles before optimising.

# 18. Materials and Manufacturing

Why Rigidity Matters for IMU Fidelity Your balance loop reads an IMU at 1kHz mounted to the chassis. If the chassis flexes at a frequency within balance loop bandwidth (0–50Hz), the IMU reads chassis vibration as actual tilt — phantom corrections result. Rigid aluminium chassis resonates above 200Hz — safely outside control bandwidth. Flexible PLA chassis can resonate at 20–80Hz — directly inside control bandwidth. This is the single highest-value reason to use a rigid chassis in ResearchBot.

## 18.1 FDM Printing

| Material | Stiffness | Impact | Heat | Creep | Verdict |
| --- | --- | --- | --- | --- | --- |
| PLA | Good static | Brittle — snaps on falls | 60°C softens | Significant — servo mounts drift | Chassis covers and non-structural only |
| PETG | Slightly lower | Better — absorbs impact | ~80°C | Reduced | Better for leg links in EduBot |
| ABS | Good | Good | ~100°C | Moderate | Marginal improvement, warps when printing |
| PA12-CF (CF nylon) | Near aluminium | Good | ~150°C | Very low | Genuine step up — use for servo mounts and leg links |

## 18.2 Resin Printing

| Type | Accuracy | Toughness | Best Use |
| --- | --- | --- | --- |
| Standard resin | ±0.05mm | Poor — brittle | Small precise brackets, sensor mounts |
| Tough/ABS-like resin | ±0.05mm | Moderate | Small structural parts |
| Formlabs Tough 2000 | ±0.05mm | Good | Leg links if resin printer available |

## 18.3 Aluminium

| Type | Rigidity | Weight | Geometry | Cost | Verdict |
| --- | --- | --- | --- | --- | --- |
| 2020 V-slot extrusion | Very high | ~0.4kg/m | Linear only | £2–4/m | Excellent for main chassis |
| Laser cut sheet 1.5–3mm | Very high in-plane | Light | Flat + tabs | £5–20/sheet JLCPCB | Excellent for base plate and mounting plates |
| Machined 6061-T6 | Best | Moderate | Any | £10–50/part CNC | Right for V2 servo mounts and leg links |

## 18.4 Carbon Fibre

| Type | Stiffness vs Al | Weight | Impact | Electrical | Cost | Verdict |
| --- | --- | --- | --- | --- | --- | --- |
| CF sheet 3mm | 3–5× stiffer at same weight | Lighter | Brittle — cracks | Conductive — keep from PCBs | £15–30/sheet | Excellent for leg links — biggest performance upgrade |
| CF tube | Very high axial | Lightest | Brittle | Conductive | £5–15/m | Useful for structural members if design calls for it |

## 18.5 Recommended Hybrid by Version

| Part | EduBot | ResearchBot V1 | ResearchBot V2 |
| --- | --- | --- | --- |
| Chassis | PLA FDM | 2020 extrusion + laser cut Al plate | Machined Al + CF panels |
| Leg links | PLA FDM | CF sheet or PA12-CF FDM | CF sheet laser cut |
| Servo mounts | PLA FDM | PA12-CF FDM 4+ perimeters | Machined 6061 Al |
| Wheel hubs | PLA FDM | PA12-CF FDM | Machined 6061 Al |
| Covers | PLA FDM | PETG FDM | PETG FDM |
| Fasteners | M3 self-tap | M3 heat-set inserts | M3 tapped into Al |

## 18.6 Where to Source

| Material | Supplier | Cost |
| --- | --- | --- |
| 2020 Al extrusion | AliExpress, Motedis | £2–4/metre |
| Laser cut Al sheet | JLCPCB sheet metal, SendCutSend | £5–20/sheet |
| Machined Al parts | JLCPCB CNC, PCBWay CNC | £10–50/part |
| CF sheet 3mm | AliExpress, Easy Composites | £15–30/300×300mm |
| PA12-CF filament | Bambu Lab, eSun, Polymaker | £40–60/kg |
| M3 heat-set inserts | AliExpress | £3–5/100 pack |
| Structural epoxy | Araldite 2011, RS Components | £8–12 |

Single Highest-Value Upgrade Replace printed chassis with 2020 extrusion + laser-cut aluminium base plate. Cost: under £15. Time: one afternoon. Result: dramatically improved IMU fidelity, better crash survivability. This single change has more impact on balance performance than any firmware tuning.

# 19. Pointers — Mental Model and Practice

## 19.1 The Mental Model

int value = 42; // a house, with the number 42 inside int* ptr = &value; // a piece of paper with that house's ADDRESS *ptr = 100; // go to that address, change what's inside to 100 & means "give me the address of" * means "go to that address and look inside" // The pointer itself is just a number — a memory address. // A 4-byte integer on a 32-bit system. Nothing magic.

The Syntax Confusion * means THREE different things: int* ptr = &value; // DECLARATION — ptr is a pointer type *ptr = 100; // DEREFERENCE — go to the address int x = 5 * 3; // MULTIPLICATION — completely unrelated Identify the context first. The confusion largely disappears.

## 19.2 Five Practice Exercises

### Exercise 1 — Observe Real Addresses

void setup() { Serial.begin(9600); int a = 10; int b = 20; Serial.println((unsigned long)&a); // a's real memory address Serial.println((unsigned long)&b); // b's address — offset from a } // Variables are not abstract — they live at specific addresses in RAM.

### Exercise 2 — Change Variable Through Pointer

void setup() { Serial.begin(9600); int value = 42; int* ptr = &value; Serial.println(value); // 42 *ptr = 100; Serial.println(value); // 100 — changed via pointer! }

### Exercise 3 — Swap Function (Most Important)

void broken_swap(int a, int b) { int temp = a; a = b; b = temp; // modifies copies — originals unchanged } void real_swap(int* a, int* b) { int temp = *a; *a = *b; *b = temp; // modifies originals } void setup() { int x = 5, y = 10; broken_swap(x, y); Serial.println(x); // still 5 — broken real_swap(&x, &y); Serial.println(x); // now 10 — works } // Write broken_swap first, observe it failing, then fix it. // This teaches EXACTLY why pointers exist.

### Exercise 4 — Pointer to Struct (Robot-Relevant)

struct SensorData { float temperature; float humidity; }; void print_data(SensorData* data) { Serial.println(data->temperature); // -> is shorthand for (*data).temperature Serial.println(data->humidity); } void setup() { SensorData reading; reading.temperature = 23.5; reading.humidity = 60.0; print_data(&reading); } // This is the exact pattern in the robot codebase: // Balance_Update(const Attitude* att, ...) — same thing.

### Exercise 5 — Hardware Register (Robot-Critical)

// PORTB controls pins 8-13 on Uno/Mega volatile uint8_t* port = &PORTB; void setup() { DDRB ¦= (1<<5); } // pin 13 as output void loop() { *port ¦= (1<<5); // pin 13 HIGH via pointer delay(500); *port &= ~(1<<5); // pin 13 LOW via pointer delay(500); } // This is how TIM3_CCR1 writes in servo.cpp work. // volatile = this memory can change without the compiler knowing.

## 19.3 Decision Framework

| Situation | Use | Why |
| --- | --- | --- |
| Function must modify original | Pointer | Copy cannot affect caller's variable |
| Data is large — struct, array | const pointer | Copying wastes stack and CPU time |
| Talking to hardware registers | volatile pointer | Fixed address in hardware memory map |
| Choose which variable at runtime | Pointer array | Index into array — one line vs switch statement |
| Small local calculation | Variable | Simple, readable, no indirection needed |
| Returning a single value | Return value | Cleaner than output pointer for single return |

"Pointers exist because functions cannot normally affect the outside world — a pointer is how you hand a function a door into the outside world when you specifically need it to."

# 20. Gait and Balance Controller Selection

## 20.1 Gait — FSM vs CPG

| Config | Right Approach | Reason |
| --- | --- | --- |
| 2 limbs, 1 DOF/leg | FSM | CPG has nowhere to go — 1 joint per limb means oscillator just drives one angle |
| 2 limbs, 2 DOF/leg (current) | FSM — CPG experiments optional | Multiple joints to coordinate. CPG starts to add value. |
| 4 limbs, 2 DOF | FSM → CPG experiments | Coupling between limb pairs meaningful. CPG worth trying. |
| 4 limbs, 3 DOF/leg | CPG strongly recommended | 12 joints — emergent multi-gait behaviour is CPG's core strength |

## 20.2 Balance — PID vs LQR vs MPC

| Controller | Pros | Cons | When to Use |
| --- | --- | --- | --- |
| Cascade PID | Tune in an afternoon, works on Mega, enormous reference material | Manual coupling, single operating point, separate velocity and balance | Always start here. Tune completely before touching anything else. |
| LQR | Handles coupling naturally, principled Q/R tuning, better disturbance rejection | Requires accurate physical model. Linear assumption breaks in walk transitions. | After PID works and robot is physically measured. |
| MPC | Handles constraints, plans ahead for gait, state of the art | Computationally expensive, hard to implement, needs very accurate model | ResearchBot V2+ with STM32H7. After LQR proven. |

PID First — Always A well-tuned PID outperforms a poorly-modelled LQR every time. LQR requires accurate measurements of mass, inertia, CoM height, wheel radius. Wrong model = wrong gains = worse than PID. Get PID working. Measure the robot. Then implement LQR. The codebase supports runtime switching: type "balance=lqr" in serial tuning terminal.

## 20.3 CPG Matsuoka Oscillator Implementation

// Half-centre oscillator — two mutually inhibiting neurons struct MO { float u1, u2; // neuron potentials float v1, v2; // fatigue variables float tau; // time constant (controls step frequency) float beta; // fatigue rate float output; // drives joint angle }; void mo_step(MO* m, float coupling, float feedback, float dt) { float du1 = (-m->u1 - m->beta*m->v1 - m->u2*(m->u2>0) + coupling + feedback) / m->tau; float du2 = (-m->u2 - m->beta*m->v2 - m->u1*(m->u1>0) + coupling + feedback) / m->tau; m->u1 += du1*dt; m->u2 += du2*dt; m->v1 += (m->u1*(m->u1>0) - m->v1)*dt/m->tau; m->v2 += (m->u2*(m->u2>0) - m->v2)*dt/m->tau; m->output = m->u1 - m->u2; } // Key params: tau (frequency), beta (adaptation), coupling weights (gait type)

## 20.4 Separate Firmware Strategy (EduBot)

| Scenario | Feasible? | Notes |
| --- | --- | --- |
| Drive firmware only | Yes — barely | 500Hz balance loop on Mega is marginal but workable |
| Walk firmware only | Yes | No balance needed — FSM servo sequencing easy on Mega |
| Both modes, never simultaneous | Marginal | Separate firmwares removes the hardest problem |
| Boot-time mode select (recommended) | Yes | Read GPIO pin at startup — one firmware, two personalities, physical switch |

# 21. Future Expansion — 4 Limbs and 3 DOF

## 21.1 What 3 DOF Unlocks

| DOF Config | Capability | Balance Approach | Gait Approach |
| --- | --- | --- | --- |
| 1 DOF/limb | Shuffle forward/backward only. No lateral correction. | Separate from gait — wheel balance | FSM optimal |
| 2 DOF (hip+knee, current) | Step forward with knee lift. | Primarily wheel balance | FSM handles well |
| 3 DOF (add hip abduction) | Full 3D foot placement. Lateral balance. Terrain adaptation. | Legs handle balance — wheels handle speed | CPG compelling — emergent gaits across 12 joints |

Standard 3-DOF per limb configuration: Hip abduction/adduction — lateral swing (NEW) Hip flexion/extension — forward/backward (current) Knee flexion/extension — up/down (current) This matches: Boston Dynamics Spot, MIT Mini Cheetah, Unitree A1/Go1 3 DOF = minimum to place foot anywhere in 3D workspace

## 21.2 Hardware Implications

| Resource | Current (2L 2DOF) | 4 Limbs 2DOF | 4 Limbs 3DOF |
| --- | --- | --- | --- |
| Servos | 4 | 8 | 12 |
| FOC motors | 2 | 4 | 4 |
| Total actuators | 6 | 12 | 16 |
| MCU | STM32F411 | STM32F405 | STM32H7 |
| Architecture | Single MCU | Dual MCU | Multi-MCU CAN bus |

## 21.3 Expansion Roadmap

| Version | Config | New Hardware | New Software | Timeline |
| --- | --- | --- | --- | --- |
| V1 | 2 limbs, 2 DOF, TT | As built | FSM, cascade PID, LQR | Now |
| V1.5 | 2 limbs, 2 DOF, gimbal | SimpleFOC Mini + GM3506 | FOC torque commands | After V1 walking |
| V2 | 4 limbs, 2 DOF | Second arm pair, STM32F405 | CPG experiments | 3–6 months post V1 |
| V3 | 4 limbs, 3 DOF | Lateral hip servo ×4, STM32H7 | Full CPG, IK, MPC | 6–12 months post V2 |

Do Not Jump to V3 From V1 V1 teaches: balance control, gait design, sensor fusion — the fundamentals. V2 teaches: CPG coupling, multi-limb coordination, hardware scaling. V3 teaches: 3D kinematics, full CPG, advanced optimal control. Attempting V3 without V1 experience means debugging 12 actuators, new MCU, new gait algorithm, and 3D kinematics simultaneously. This reliably fails.

# 22. Racing — Wheeled Mode at Speed

What Racing Means for This Robot Racing is wheeled drive only — no gait. The robot is a fast segway. Servos are used to position legs in an optimised pose and hold it during racing. Lower CoM height (tucked legs) = more stable at speed. Wider effective wheelbase (legs angled outward) = higher speed stability margin. The balance controller still runs — just with higher gains and different parameters. This is distinct from dynamic legged racing which requires completely different hardware.

## 22.1 What Changes in Race Mode

### Leg Pose Manager

In race mode the gait FSM is replaced by a leg pose manager that moves legs to a pre-computed racing pose and holds it:

// Race pose: legs tucked to lower CoM, improve aerodynamics static const LegPose POSE_RACE = { .hip_left_deg = 15.0f, // lean legs slightly forward at speed .hip_right_deg = 15.0f, .knee_left_deg = 25.0f, // tuck knees — lowers CoM from 12cm to ~8cm .knee_right_deg = 25.0f, }; // In MODE_RACE: Gait_Update() moves to POSE_RACE via interpolation // then holds — no stepping, no state transitions // Balance loop continues at 1kHz as normal

### Higher Gain Balance Tuning

At speed the inverted pendulum dynamics change — the robot needs faster corrections. Race mode switches to a separate gain set:

// Race mode gains in config.h #define RACE_INNER_KP 150.0f // ~2x normal drive KP #define RACE_INNER_KD 20.0f // more damping at speed #define RACE_INNER_KI 3.0f #define RACE_OUTER_KP 8.0f // faster velocity response #define RACE_OUTER_KI 0.3f #define RACE_MAX_VEL 3.0f // m/s — set this conservatively at first // Switch gains in ISR when mode changes: // triggered by L2 button hold on controller

## 22.2 Why CoM Height Matters for Racing Speed

The maximum stable speed of a wheeled inverted pendulum scales with CoM height:

// Simplified stability limit: // Max angular acceleration the motors can provide must exceed // the gravitational torque about the wheel contact point. // Lower CoM = smaller gravitational torque = faster possible correction // Lower CoM = smaller perturbation from the same disturbance // Approximate max stable speed (m/s) for wheeled pendulum: // v_max ≈ sqrt(g × wheelbase / (2 × CoM_height)) // At CoM = 12cm (standing): v_max ≈ sqrt(9.81 × 0.15 / 0.24) ≈ 2.5 m/s // At CoM = 8cm (race tuck): v_max ≈ sqrt(9.81 × 0.15 / 0.16) ≈ 3.0 m/s // Tucking legs gives ~20% speed increase for free

## 22.3 Long Range Wireless — ELRS

### Why ELRS is the Correct Choice

| Option | Latency | Range | Cost | Racing Verdict |
| --- | --- | --- | --- | --- |
| ELRS 2.4GHz | 2ms at 1000Hz | 5–30km | £25 total | Best — designed for racing, hardware failsafe |
| ELRS 900MHz | 4ms at 500Hz | 10–50km | £30 total | Best for extreme range, slight latency penalty |
| Traditional RC (FlySky) | 8–22ms | 1–2km | £40–80 | Too slow for racing, too short for long range |
| WiFi (ESP32 direct) | 10–50ms | 100m | £0 | Completely unsuitable |
| LoRa | 50–200ms | 10–50km | £15 | Range yes, latency no — not for racing |
| Bluepad32 BT | 10–20ms | ~10m | £0 extra | Short range practice only |

Why 2ms Latency Matters for Racing At 3 m/s, a 20ms latency means the robot has moved 6cm before your input arrives. For balance corrections at speed this is catastrophic — the robot has already fallen. ELRS 2ms latency means only 0.6cm of travel before correction — acceptable. Never use WiFi or standard Bluetooth for racing speed control.

### ELRS Integration Architecture

Physical ELRS receiver → UART (CRSF protocol) → ESP32-S3 ↓ Parse CRSF packet Extract channels Build 9-byte robot packet ↓ UART 460800 baud ↓ STM32F411 comms.cpp → balance controller setpoints // Same packet format as Bluepad32 path — STM32 cannot tell the difference // ESP32 is a transparent relay in ELRS mode

### CRSF Packet Parsing on ESP32

// CRSF packet: 26 bytes from ELRS receiver // 16 channels packed as 11-bit values (0–2048) #define ELRS_SERIAL Serial1 // GPIO17 TX, GPIO18 RX #define ELRS_BAUD 420000 // CRSF fixed baud rate struct CRSFChannels { uint16_t ch[16]; // 0=1000 (min), 1024=centre, 2047=max }; float map_channel(uint16_t raw, float min_out, float max_out) { return min_out + (raw - 1000.0f) / 1047.0f * (max_out - min_out); } // Channel mapping: // CH1 (left stick Y): velocity → -3.0 to +3.0 m/s in race mode // CH2 (right stick X): yaw rate → -2.0 to +2.0 rad/s // CH5 (switch A): mode → 1000=drive, 1500=walk, 2000=race // CH6 (switch B): failsafe → used by ELRS hardware failsafe

### ELRS Failsafe Configuration

Hardware failsafe is non-negotiable for racing. Configure this before first range test:

// In ELRS configurator (expresslrs.org): // Failsafe mode: "Set position" // CH1 (velocity): 1024 → 0 m/s (stopped) // CH2 (yaw): 1024 → 0 rad/s (straight) // CH5 (mode): 1000 → drive mode (not race — reduces speed on failsafe) // CH6: 1000 → acknowledged // When receiver loses signal: // 1. ELRS hardware immediately outputs above preset values (<2ms) // 2. ESP32 comms watchdog zeros commands after 200ms if CRSF stops // 3. STM32 CONTROLLER_TIMEOUT_MS zeros commands after 200ms independently // Three independent safety layers

## 22.4 Motor Selection for Racing

Wheeled racing requires motors that operate efficiently at higher wheel speeds. Gimbal motors (80KV) operate at ~4% of their speed range at racing speeds — FOC is unreliable at this extreme.

| Motor | KV | At 4S Free RPM | At 2 m/s (65mm) RPM | % Operating Range | Racing Verdict |
| --- | --- | --- | --- | --- | --- |
| GM3506 gimbal (current) | 80KV | 1184 RPM | 590 RPM | 50% — acceptable to 2 m/s | Adequate if top speed ≤2 m/s |
| iPower GM4108H | 120KV | 1776 RPM | 590 RPM | 33% — good | Best walk+race compromise motor |
| T-Motor MN2204 | 1400KV | 20720 RPM | 590 RPM | 3% — unusable | Too high KV — needs large reduction |
| MN1004 + 3:1 reduction | 400→133KV eff. | 5920→1973 RPM eff. | 590 RPM | 30% — good | Best racing performance with gearbox |
| 5008 + 10:1 cycloidal | 330→33KV eff. | 4880→488 RPM eff. | 590 RPM | 24% — marginal | Now viable for racing — cycloidal justified here |

Motor Strategy by Version V1 (up to 2 m/s): GM3506 works — already in design, no change needed. V1.5 (up to 2.5 m/s): iPower GM4108H-120T — best walk+race compromise, drop-in replacement. V2 (up to 3+ m/s): MN1004 400KV with 3:1 reduction, or 5008 + 10:1 cycloidal hub. Do not change motors until V1 is walking and balancing reliably.

## 22.5 Battery Upgrade for Racing

| Parameter | Walk Config (3S) | Race Config (4S) | Reason |
| --- | --- | --- | --- |
| Voltage | 11.1V nominal | 14.8V nominal | Higher voltage = lower current for same power |
| Capacity | 1000–1500mAh | 3000–4000mAh | Racing draws 3–4× more current than walking |
| C rating | 20C adequate | 30C minimum | Peak current must not sag voltage rail |
| Peak current (walk) | ~8A total | — | TT motors + servos |
| Peak current (race) | — | ~25–40A total | Higher speed = more motor current |
| Connector | XT30 | XT60 minimum | Higher current needs larger connector |
| Weight | ~80g | ~200g | Accounts for in balance controller CoM update |

CoM Shift with 4S Battery A 4S 4000mAh LiPo is significantly heavier than a 3S 1000mAh pack. Remeasure COM_HEIGHT_M after fitting the racing battery. Update the LQR model with new CoM height before tuning race mode gains. Mount battery as low as possible — below the wheel axis if chassis allows.

## 22.6 Long Distance Operation — GPS and FPV

### GPS Module — u-blox M9N or BN-880

| Feature | Detail |
| --- | --- |
| Position accuracy | 2–5m CEP — sufficient for geofencing and RTH |
| Update rate | 10–20Hz — fast enough for navigation |
| Interface | UART — one port on ESP32 |
| Compass | BN-880 includes integrated magnetometer — provides absolute heading |
| Mounting | Away from motors and power cables — magnetic interference corrupts heading |

GPS enables three critical capabilities:

- Position telemetry: Stream GPS coordinates back to ground station over WiFi/ELRS.

- Return to home: If signal lost >1000ms, ESP32 navigates robot back to launch GPS coordinate at reduced speed.

- Geofencing: If GPS position exceeds maximum radius, ESP32 caps velocity setpoint to 0.5 m/s and issues audible warning.

### FPV for Visual Range Extension

| System | Latency | Range | Resolution | Cost | Notes |
| --- | --- | --- | --- | --- | --- |
| Walksnail Avatar HD | 4ms | 2–10km | 1080p | £80–120 | Best value HD FPV for this application |
| DJI O3 Air Unit | 4ms | 10km | 1080p | £150–200 | Best range and quality — higher cost |
| Analog (Foxeer/RunCam) | 1–2ms | 500m–2km | 480–720p | £20–40 | Cheapest, lowest latency, lower range |

Camera mounting: Forward-facing on the robot's front, tilted 5–10° upward to see terrain ahead during fast forward movement. Mount on a soft-mount pad to reduce vibration in the video feed.

## 22.7 Speed and Safety Architecture

### Layered Safety for Racing

Layer 1 — ELRS hardware failsafe (<2ms) Receiver detects signal loss → preset stop channel values immediately Physically in ELRS receiver hardware — cannot be disabled by software bug Layer 2 — ESP32 CRSF watchdog (200ms) If no CRSF packet for 200ms → send zero velocity/yaw to STM32 Layer 3 — STM32 command timeout (200ms) If no valid packet from ESP32 → zero all setpoints Independent of ESP32 — MCU fault does not prevent this Layer 4 — Tilt cutoff (35°) IMU reads excessive tilt → Motor_Coast() immediately Prevents violent tumbling at speed Layer 5 — GPS geofence Outside boundary → velocity cap 0.5 m/s Does not stop robot — allows controlled return Layer 6 — Return to home Signal lost >1000ms → navigate to home GPS at 0.5 m/s Requires GPS + basic navigation on ESP32

## 22.8 Racing Speed Development Sequence

- Walk mode working and stable. Do not attempt racing until walking is reliable.

- Drive mode well tuned. PID or LQR stable at normal speeds (0.3–0.5 m/s).

- Race pose validated. Command POSE_RACE statically with robot on stand. Verify legs reach target angles safely.

- ELRS bound and failsafe confirmed. Test failsafe at zero speed: remove antenna, verify robot stops.

- GPS geofence active. Set 10m radius. Verify robot slows at boundary.

- First race mode test at 0.5 m/s. Incremental speed increase. Characterise balance at each step.

- Tune race mode gains. Use serial tuning interface: increase RACE_INNER_KP until oscillation, back off 30%.

- Increase speed limit in 0.5 m/s increments. Stop when behaviour degrades.

- Upgrade battery to 4S when 3S limit reached. Remeasure CoM before continuing.

- Motor upgrade for higher top speed. Replace with higher KV motor when current motor at 50%+ range.

## 22.9 Expected Performance

| Configuration | Expected Top Speed | Limiting Factor |
| --- | --- | --- |
| V1: GM3506 + 3S + standing legs | 1.0–1.5 m/s | Motor operating range (too slow) |
| V1 race mode: GM3506 + 3S + tucked legs | 1.5–2.0 m/s | Balance gain bandwidth |
| V1.5: GM4108H + 4S + tucked legs | 2.0–2.5 m/s | Balance gain bandwidth |
| V2: MN1004 + 3:1 reduction + 4S | 3.0–4.0 m/s | Wheel slip and CoM stability |
| V2.5: 5008 + cycloidal + 4S | 2.5–3.5 m/s | Cycloidal efficiency and weight penalty |

Racing Research Opportunities Characterise how optimal PID/LQR gains change with forward speed — publish as gain schedule. Measure maximum stable speed as function of CoM height — validate against pendulum physics model. Compare ELRS latency at different packet rates and its effect on balance stability. Study leg pose optimisation: which tuck angle gives highest stable speed?

End of Project Reference Document — v3.0  |  22 Sections